## HHR (2008 - 2009) (2011) TPMS Reset Procedure

## TECH 2 METHOD

- Connect the GM Tech 2 scan tool

- Turn ON the ignition with the engine OFF

- Power On the Scan Tool

- Hit Enter

- Main Menu > F0: DIAGNOSTICS

- Vehicle identification – Model Year > (6) 2006

- Vehicle identification – Product Type > Passenger Car

- Vehicle identification – Product Make > (1) Chevrolet

- Vehicle identification - Product Line > A

- Diagnostics > F1: Body

  - (Establishing Communication)

- Body> F5: Remote Control Door Lock Receiver

- Remote Control Door Lock Receiver > F5: Module Setup

- Module Setup > F4: TPMS Learn Mode

  - (Establishing Communication)

- LEARN

  - (Horn Chirps Twice)