## HHR (2008 - 2009) (2011) TPMS Reset Procedure

## KEY FOB METHOD

- Apply parking brake. Turn ignition to ON position (engine off)

- Press and hold unlock and lock buttons on the keyless entry transmitter until horn sounds, and then LF turn signal will become illuminated.

- Starting with LF tire, increase/decrease tire pressure until horn sounds.

- After horn sounds proceed as in step 3 for the next 3 sensors in the following order: RF RR LR After completing LR tire horn will sound twice.

- Turn ignition to OFF position to exit vehicle learn mode. Adjust all tires to pressure listed on the placard.